CCNA 3: LAN Switching and Wireless

Chapter 7: The Wireless LAN

Lesson: Wireless Standards

Aim: How do wireless standards determine the capacity of today’s networks?

H.W.: Explain the security risks wireless networks are susceptible to and the various defenses used to combat these exploits?

Do Now: Define the following terms:

802.11a

802.11b

802.11g

802.11n

Objectives:

- Analyze the different wireless standards that are available.

- Compare the features that each standard offers.

Motivation: How do wireless networks differ from wired networks?

Wired

- It requires a cable to be run to every device on the network.

- Dedicated bandwidth is available.

- Security is easier to administer.

- Devices are stationary.

Wireless

- Location of devices are flexible

- No cables required (reduces the cost of wiring infrastructure)

Question: What are the economic advantages of implementing a wireless network?

Question: Explain the disadvantages of wireless networks.

- Broader range of devices can connect to a network.

- Mobility

- Security is harder to enforce.

- Bandwidth is shared by users.

802.11a

- 802.11 was the original wireless protocol and ratified in 1997

- 802.11a was ratified in 1999

- 5 GHz

- 54 Mbps

- 35 meters

Question: How do you convert meters into feet? Multiply the number of meters by 3.2808.

Question: How many feet is 35 meters? 150 ft.

802.11b

- Ratified in 1999

- 2.4 GHz

- Up to 11 Mbps

- 35 meters

Question: Why did the wireless standards change frequencies? The 5 GHz signal cannot penetrate walls and certain structures. The 2.4 GHz signal is not affected by walls and structures, but is susceptable to interference from household appliances that operate on the same frequency, such as cordless phones and microwaves.

802.11g

- Ratified in June of 2003

- 2.4 GHz

- Up to 54 Mbps

- 70 meters

Question: Why is 802.11g backwards compatible with 802.11b, but incompatible with 802.11a? 802.11g and 802.11b operate on the same frequency.

802.11n

- 2.4 and 5 GHz

- Is backwards compatible with 802.11a, b, and g standards.

Question: Why is 802.11n backwards compatible with the a, b, and g standards? It operates on both frequencies.

- Up to 300 Mbps but usually only supports 100 Mbps (same as most wired networks)

- 230 ft

Question: How do you convert feet into meters? Multiply the number of feet by .3048.

Question:  How many meters is 230 feet? 70 meters.

- MIMO technology (Multiple-Input, Multiple-Output)

- Uses different antennas to send and receive data

- Increases throughput and accomplishes more of a full duplex operation

- Violates the “Good Neighbor Policy”

Note: Blasting your Wi-Fi signal into a neighbor's apartment or house can interfere with their connection (slowing down their web surfing or even disconnecting them entirely). Wi-Fi signals are radio signals, so the problem is similar to the way that a microwave might interfere with a cordless phone, resulting in cut-out calls or static-filled reception. Just like blasting your radio, excessive wireless signal strength violates the Good Neighbor Policy.

Summative Questions

Why are different standards used for wireless?

Which standards are used for enterprise and home networks?

How does range affect connectivity?

Why is it important for standards to support roaming?